CS_113

## FROM LANGUAGES TO HARDWARE

(Attempt 2 questions out of 3)

Question 1.

- Undertake the following calculations:

- Convert the binary number 11100111 to base 10.

- Convert the binary number 11100111 to hexadecimal.

- Convert 46 base 10 to binary coded decimal (BCD).

- Find the binary two’s complement representation of –5 (i.e. negative 5).

- The binary number 10010110 is shifted one place to the right so the least significant digit is lost and a zero is inserted at the most significant end. Determine the resulting value – express your answer in base 10 and state the mathematical operation which occurs when a binary number is shifted in this way.

[5 marks]

- A microprocessor is servicing an interrupt when a second interrupt occurs from a different source. Describe IN DETAIL the events that may occur should this situation arise.

[10 marks]

- Describe in detail each of the following:

- The operation of a Cathode Ray Tube based computer display.

- The frame sequential stereoscopic display.

- In the case of the frame sequential stereoscopic display, what additional demand does this place upon the Cathode Ray Tube?

[10 marks]

Question 2.

- By means of appropriate circuit diagrams, explain clearly the operation of each of the following:

- A level triggered D-type.

- An edge triggered Master Slave D-type.

[5 marks]

- Describe IN DETAIL the benefits that may be derived through pipelining instructions within a microprocessor, AND discuss three types of pipeline hazard. For each of these hazards you should describe briefly how the impact of the hazard may be reduced.

[10 marks]

- Simplify the following Boolean expression algebraically.  You should clearly state any Boolean propositions or theorems used.

# A  .  B +  A  .  B +  A .  B

[10 marks]

Question 3.

- Charles Babbage is often referred to as the pioneer of the modern computer system.

- Explain one major difference between Babbage’s Difference Engine and his Analytical Engine.

- Describe three key architectural features that Babbage proposed and adopted in his designs, that mimic the architecture of today’s computer systems.

[5 marks]

- A clock generates a 10MHz digital signal. Calculate the period of the waveform produced?

[5 marks]

- Describe IN DETAIL the events which occur when a microprocessor is reset.

[5 marks]

- Today’s computers support multi-tasking. Discuss in detail how this may be achieved.

[10 marks]

### END OF PAPER